INVITATION TO BID

TENDER for Supply of Material and Construction of 30 Toilets & Repairment

Budgam, Jammu and Kashmir- India

TENDER ADVERTISMENT: May 13, 2026.

Tender No. Care/02/2026

TENDER DOCUMENT ISSUANCE DATE: May 13, 2026

To May 19, 2026 EOD

LAST DATE FOR DOCUMENTS SUBMISSION: May 19, 2026

By 05:30 PM

INVITATION TO BID / TENDER NOTICE

CARE World invites sealed quotations/bids from eligible and experienced construction firms/contractors for the construction of school toilet units under its WASH and Education support activities in Jammu & Kashmir.

Project Title

Empowering Education Through Inclusion , Infrastructure Community Engagement in Budgam District. Project Pin: HQ0202757101

Scope of Work

The selected contractor/ firm shall be responsible for construction of toilet units including provision of construction materials, labor, technical supervision, transportation, tools, and all associated construction activities as per approved specifications and standards.

The work includes:

- RCC foundation and plinth

- Brick/cement brick masonry and cement plaster

- Anti-skid floor tiling

- CGI sheet/RCC roofing

- Installation of doors and ventilation

- Water connection and drainage system

- Septic tank/soak pit arrangement

- Child-friendly and gender-segregated toilet design

Eligibility Criteria

Interested bidders should:

- Be registered construction firms/contractors

- Have relevant experience in civil/construction works

- Possess valid registration and tax documents

- Demonstrate capacity to complete work within the specified timeline

Bidding option B

CARE World also encourages suppliers who can only provide Material Separately, and Firm Provides Only Labor/Technical Services

In this model:

- CARE World purchases construction material directly

- Firms/Individual contractors only provides:

  - Mason

  - Helpers

  - Technical advisor

  - Execution/service

- Billing is only for labor/service charges

Tender Documents

Interested bidders may obtain the detailed tender documents from the CARE World office or through official communication channels during office hours.

Submission of Bids

Completed bids in sealed envelopes clearly marked at the Care World Office: Chanapora, near Abdullah filling station, Near Iqra Public School Srinagar, Jammu & Kashmir – 190015, India

“Tender for Construction of School Toilet Units” must be submitted on or before 19 / 05/ 2026 at 05:30 PM.

Opening of Bids

Bids will be opened in the presence of procurement representatives on 20/ 05 / 2026 at CARE World office followed by Procurement committee analysis and final selection of the contractor/supplier/firms

CARE World reserves the right to accept or reject any or all bids without assigning any reason thereof.

For further information, please contact: tenders@careworld.org.in

CONTENTS

- Supplier’s general business info

- Pricing (RFQ)

- Terms and Conditions

- Supplier Screening Form

- Queries	write us

- Complaint…. write us

- Supplier Code of Conduct

*Please take print out of this and fill the information

SUPPLIER INFORMATION

Company Information

| Question | Responses |
| --- | --- |
| Full Legal and Trading Name. |  |
| Date of Registration of your company (please attach certificate) |  |
| Full address. |  |
| Registered place of business (if different from street address). |  |
| Name the contact within your organization to whom all correspondence regarding this should be addressed. This include Cell#, Email Address |  |
| Company website address (if any). |  |
| Name and position of person authorized to enter into negotiations and sign any formal agreement. |  |
| Company registration details such as, - Registration numbers - GST Registration number - Trade license number (copy must be attach) |  |

  - Business Information

| Question | Response |
| --- | --- |
| List of Products/Services |  |
| List of Major Clients |  |
| Details of your company’s experience in the manufacture or supply of similar items, keeping in view the following points: - Name of company/NGOs - Contract Value - References |  |

| Question | Response |
| --- | --- |
| Bank Name, Account Title Bank Account name should be the same as the supplier business name |  |

Banker’s Detail:

  - Pricing

M/s		Date:

Request for Quotation

Care World takes this opportunity to request you to quote the prices for the Supply of Material and Construction of toilets in Schools as per following details:

|  | Twin latrines with hand-washing unit (5'x6') with Septic Tank & Soakage Pit |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S# | Description | Unit | Qty. | Unit rate | Amount |
| 1 | EXCAVATION |  |  |  |  |
|  | Excavation in any type of soil, including back filling in excavated area and disposal of surplus un-suitable material |  | 30 |  |  |
| 2 | Material Required |  |  |  |  |
|  | Cement (Bags) | 50 kg/bag |  |  |  |
|  | Bricks | Unit |  |  |  |
|  | Sand (Cubic feet) | CFT 100 |  |  |  |
|  | Aggregates (Cubic feet) | CFT |  |  |  |
|  | Steel (kg) | kg |  |  |  |
|  | PVC Pipes (4-inch) | Feet |  |  |  |
|  | Water Storage Tank (500L) | Nos. |  |  |  |
|  | Toilet Pan & Fittings (including flushing) | Set |  |  |  |
|  | Door (Iron/Wooden) | Nos. |  |  |  |
|  | Tiles (Floor & Walls) half | Sq. ft. |  |  |  |
|  | Water Tap & Piping | Nos. |  |  |  |
|  | Roofing Slab | Sq. ft. |  |  |  |
|  | Ventilation Grills or Small Windows | Sq. ft. |  |  |  |
|  | Paint & Finishing | Sq ft |  |  |  |
|  | Site Preparation & Excavation | Sq |  |  |  |
|  | Drainage Pit or Septic Tank | Ft |  |  |  |
|  | Water Filter non electric | Nos |  |  |  |
|  | Water Filter water connection Pipes | Nos |  |  |  |
| 4. Job Description For 1 Mason, 2 Helpers and 1 Technical Advisor Technical Staffing Requirements The contractor/vendor shall deploy qualified and experienced personnel for the execution and supervision of toilet construction works. The minimum required technical staff shall include: S. No.; Position; Minimum Requirement; Responsibilities. 1; Mason; 1 Skilled Mason; Responsible for masonry work, toilet structure construction, plastering, brick/block laying, flooring, and finishing works as per approved design and quality standards.. 2; Skilled Helpers; 2 Skilled Helpers; Assist the mason in material handling, mixing, excavation support, curing, installation support, cleaning, and other construction-related activities.. 3; Technical Advisor / Site Supervisor; 1 Technical Advisor; Responsible for technical oversight, quality assurance, adherence to drawings/specifications, measurement verification, progress monitoring, and compliance with safety and project standards.. Additional Conditions - All deployed staff must have relevant experience in construction works. - The contractor shall ensure proper safety measures and supervision at the construction site. - Replacement of technical staff during implementation shall only be made with personnel of equivalent qualification and experience. - The Technical Advisor shall coordinate with the project team for inspections, monitoring, and approvals. S. No.; Position; Unit; Qty; Rate (Unit); No. of Days; Total Amount. 1; Mason; 1 Toilet; 1; INR; INR. 2; Skilled Helper; 1 Toilet; 2; INR; INR. 3; Technical Advisor / Site Supervisor; 1 Toilet; 1; INR; INR. Please Note - Rates quoted shall be on a contractual basis and shall remain fixed irrespective of the number of working days involved in completion of the work. - Bidders shall clearly mention the approximate number of days required for completion of the assigned work. - Evaluation and selection may also consider the proposed completion timeline in addition to the quoted financial rates. - Quoted rates shall include labor charges, supervision, transportation, tools, equipment, and all associated construction activities. |
| 2 | Excavation |  |  |  |  |

5.DESCRIPTION OF WORK

Recommended Standard School Toilet Dimensions and Construction Specifications

1. Student Toilet Unit Dimensions

| Component | Recommended Specification |
| --- | --- |
| Internal Width | 3 ft – 4 ft (900 mm – 1200 mm) |
| Internal Length | 4 ft – 5 ft (1200 mm – 1500 mm) |
| Clear Height | Minimum 7 ft – 8 ft |
| Door Width | Minimum 2.5 ft (750 mm) |
| Ventilator Size | 1.5 ft × 1 ft |
| Wall Thickness | Minimum 4.5-inch brick wall |
| Floor Slope | Proper slope toward drain outlet |
| Roof Height | Approximately 8 ft from floor level |

2. Accessible Toilet for CWSN (Children With Special Needs)

| Component | Recommended Specification |
| --- | --- |
| Internal Size | Minimum 5 ft × 7 ft |
| Door Width | Minimum 900 mm |
| Ramp Width | Minimum 1.2 m |
| Ramp Slope | 1:12 |
| Wheelchair Turning Space | Minimum 1500 mm turning radius |
| Handrails | Provided on both sides |
| Door Opening | Outward opening preferred |

3. Septic Tank Dimensions (Approximate)

| Toilet Capacity | Recommended Septic Tank Size |
| --- | --- |
| 1–2 Toilet Seats | 6 ft × 4 ft × 5 ft depth |
| 3–4 Toilet Seats | 8 ft × 5 ft × 6 ft depth |

4. Urinal Section (Boys)

| Component | Recommended Specification |
| --- | --- |
| Width per Urinal | 2 ft |
| Divider Height | 3 ft – 4 ft |
| Drainage | Proper floor slope toward drain outlet |

5. Handwashing Area

| Component | Recommended Specification |
| --- | --- |
| Counter Height | Approximately 2.5 ft for children |
| Number of Taps | Minimum 2 taps |
| Drainage System | Connected to soak pit/drain line |

6. Recommended Construction Specifications

| Component/Item | Recommended Specification |
| --- | --- |
| Foundation & Plinth | RCC foundation and plinth as per site conditions |
| Masonry Work | Brick masonry in cement mortar |
| Plastering | Cement plaster inside and outside |
| Flooring | Anti-skid floor tiles |
| Roofing | CGI sheet or RCC roofing |
| Doors | PVC or metal frame doors with locking arrangement |
| Ventilation | Adequate ventilation and natural lighting |
| Water Supply | Provision for water storage and supply connection |
| Drainage System | Proper drainage with soak pit/septic tank arrangement |
| Design Features | Child-friendly and gender-segregated layout |
| Accessibility | At least one accessible toilet for CWSN students |

6. Toilet layout Work

| S. No. | Item | Standard Specification |
| --- | --- | --- |
| 1 | RCC Foundation and Plinth | Reinforced Cement Concrete (RCC) foundation and plinth shall be constructed as per site condition and soil bearing capacity using standard cement, sand, and aggregate mix. Minimum plinth height should be 1–1.5 ft above ground level to prevent water seepage and snow/rain impact. |
| 2 | Brick Masonry in Cement Mortar | Brick masonry shall be constructed using first-class burnt bricks in cement mortar ratio 1:6. Wall thickness shall be minimum 4.5 inches for partition walls and 9 inches for external walls where required. |
| 3 | Cement Plaster (Inside & Outside) | Internal and external wall surfaces shall be plastered with cement mortar (1:4 ratio) with smooth finish. Internal plaster thickness shall be minimum 12 mm and external plaster 15 mm. |
| 4 | Anti-Skid Floor Tiles | Toilet floors shall be finished with anti-skid ceramic/vitrified tiles suitable for wet areas. Tiles shall be durable, easy to clean, and installed with proper slope toward drainage outlets. |
| 5 | CGI Sheet / RCC Roofing | Roofing shall consist of either RCC slab or CGI sheet roofing with proper structural support, waterproofing, and slope for rain/snow drainage suitable for climatic conditions of Jammu & Kashmir. |
| 6 | PVC Door or Metal Frame Door with Latch | Each toilet unit shall have PVC/FRP or metal-framed door with internal latch/locking system. Door width shall be minimum 750 mm and designed for child-friendly use and privacy. |
| 7 | Ventilation and Natural Lighting | Adequate ventilation shall be provided through ventilators/windows to ensure airflow, odor control, and natural lighting. Ventilator size should be approximately 1.5 ft × 1 ft minimum. |
| 8 | Water Storage Connection | Toilet units shall be connected to an adequate water source/storage system including water tank, supply pipes, taps, and flushing arrangements to ensure uninterrupted water availability. |
| 9 | Proper Drainage and Soak Pit/Septic Arrangement | Proper wastewater drainage system shall be constructed and connected to soak pit or septic tank as per approved sanitation standards. All pipelines shall ensure leak-proof disposal and environmental safety. |
| 10 | Child-Friendly and Gender-Segregated Design | Separate toilet units shall be constructed for boys and girls with child-friendly dimensions, privacy arrangements, easy access, proper signage, and safe usage facilities suitable for school children. |

Note: Care World will have the right to conduct the test for of samples to evaluate the quality and also the Consignment if selected.

7. Delivery

| Delivery Time | Mention Delivery Date ……………………………………. |
| --- | --- |
| Delivery Location | District Budgam, 30 Villages in Block Chari-sharief, Chadooora, Sursyaar, Nagam, Khansahab, Pakherpora Jammu and Kashmir – India |

- General Terms & Conditions (Failure to comply with any of the terms and conditions mentioned in this tender document may result in disqualification of the bidder)

      - Quoted prices for the above items shall be inclusive of all kinds of Govt. taxes and duties as per prevailing Tax Laws of Govt. of Jammu and Kashmir- india,

      - Payment shall be made in the form of a Cheque within 21 days after execution of the order and subsequent submission of the invoice to the Care World

      - Any damage, loss, theft and demurrages before delivery at final destination will be responsibility of supplier.

      - Evaluation of the quotation and award of Purchase Order shall be product wise.

      - Care World reserves the right to reject any or all quotations without assigning any reason thereof.

      - Expected delivery time must be mentioned in your quotation.

      - The contractor/construction firm shall carefully examine the tender documents, specifications, drawings, and site conditions before submission of the bid.

      - Submission of a bid shall be considered as confirmation that the bidder has fully understood and accepted all terms and conditions of the tender.

      - The selected contractor shall complete the construction works within the agreed timeline mentioned in the contract/work order.

      - All construction materials used shall be of approved quality and in accordance with the technical specifications provided by CARE World.

      - The contractor shall provide all required labor, technical supervision, tools, equipment, transportation, and associated resources necessary for successful completion of the work.

      - The contractor shall ensure deployment of minimum required personnel, including:

1 Mason

2 Skilled Helpers

1 Technical Advisor (Optional this can be of Care World hired separately)

      - The contractor shall maintain proper workmanship, safety standards, and cleanliness at the construction site throughout the implementation period.

      - Any defective work or use of substandard materials identified during inspection shall be rectified/replaced by the contractor at their own cost.

      - CARE World reserves the right to inspect the work at any stage of implementation and issue necessary instructions for quality assurance and compliance.

      - Payments shall be made based on verified completed work, approved measurements, and submission of original invoices/bills.

      - No advance payment shall be made unless otherwise approved in writing by CARE World.

      - The contractor shall be responsible for any damage, accident, or loss occurring at the construction site during execution of the work.

      - Delays in completion without valid justification may result in penalties, deduction in payment, or cancellation of the contract.

      - The contractor shall comply with all applicable local laws, labor regulations, environmental standards, and safety requirements.

      - The contractor shall not subcontract the work to another party without prior written approval from CARE World.

      - CARE World reserves the right to accept or reject any bid wholly or partially without assigning any reason.

      - In case of any dispute, the decision of CARE World shall be considered final and binding.

      - The contractor shall hand over the completed toilet units in clean, functional, and usable condition before final approval and payment.

      - All quoted rates shall remain fixed during the contract period and shall include all taxes, transportation, labor, supervision, tools, equipment, and associated construction costs unless otherwise specified.

      - Bidders must clearly mention the approximate timeline required for completion of the work in their financial/technical proposal.

Sealed Quotations addressed to “Procurement Committee (Care World) Clearly Written tender Title should be dropped in our quotation box placed at Care World- Chanapora Bypass- Near Abdullah Filling Station, Near Iqra Public School- Srinagar Chadoora Road

Validity of Bid

      - Bids shall remain valid for at least 90 days from the date of submission. If the last date falls on a holiday, the validity shall be extended to the next working day of the Company thereafter.

Supplier Screening Form

Company Information:-				 Company Name (as appeared on Registration Document): 			  Company Address: 			 Address:	Telephone:	Website: 	  Owner /Director(s) names (Please ensure you enclose proof of identity of director(s) e.g. passport copy or ID card)

Has the organization been convicted of any criminal offence?

Yes	No

Does the organization have any relationship with current care World  in the following capacity?

| Personal/Family | Yes | No |
| --- | --- | --- |
| Business | Yes | No |

If you have answered YES to any of the above then can you please state in detail the relationship you have and with whom

How did you hear about Care World’s service request? 	 Please note that it is compulsory for the following to be provided to us:

- Company Registration

- Photo copy of ID/Passport of Directors

Consent

Our organization is not involved in and does not support any activity which is considered illegal by the Government of India or under Community or what may be termed a ‘terrorist activity’

I confirm that the above information is accurate to the best of my knowledge. I have not withheld information.

Name:	Position: 		 Sign & Official stamp: 		Date: 	_

We will treat your personal information as confidential and your details will not be shared with anyone else. The

information on this form is required for the purpose of providing security screening.

  - Queries……… please write us

If you have any query on tender document than please write to

info@careworld.org.in

Tenders@careworld.org.in

  - Complaints……… please write us

If you see any violation of rights or any incident of corruption, please contact us at:

complaints@careworld.org.in

CARE World has zero tolerance for corruption & bribery and is committed to listen and Address any violation of rights of humanitarian workers, suppliers, contractors and our beneficiary communities.

SUPPLIER CODE OF CONDUCT

Supplier / Contractor Code of Conduct

All suppliers, contractors, vendors, and construction firms participating in this tender process shall comply with the following Code of Conduct and all applicable laws and regulations of India.

1. Compliance with Laws

The supplier/contractor shall comply with all applicable laws, rules, and regulations of the Government of India and the Union Territory of Jammu & Kashmir, including labor laws, taxation laws, environmental regulations, construction standards, and occupational safety requirements.

2. Ethical Conduct

The supplier/contractor shall conduct business in an ethical, transparent, and professional manner and shall not engage in any form of fraud, corruption, bribery, collusion, coercion, or misrepresentation during the tendering and implementation process.

3. Child Labour Prohibition

The supplier/contractor shall strictly prohibit the use of child labor in accordance with:

- The Child and Adolescent Labour (Prohibition and Regulation) Act, 1986 (as amended)

- Applicable Indian labor laws and international humanitarian standards

4. Forced Labour Prohibition

The supplier/contractor shall not engage in forced, bonded, compulsory, or exploitative labor practices under any circumstances.

5. Non-Discrimination

The supplier/contractor shall ensure equal treatment and non-discrimination on the basis of gender, religion, caste, disability, ethnicity, or social background in recruitment and workplace practices.

6. Workplace Safety

The supplier/contractor shall provide a safe and secure working environment for all workers and comply with occupational health and safety standards, including:

- Use of protective equipment where necessary

- Safe handling of construction materials and tools

- Site safety and accident prevention measures

7. Prevention of Sexual Exploitation, Abuse, and Harassment (PSEAH)

The supplier/contractor and all deployed personnel shall maintain professional conduct and shall not engage in:

- Sexual exploitation or abuse

- Harassment or intimidation

- Exploitative behavior toward children, beneficiaries, or community members

Any violation may result in immediate termination of the contract and legal action.

8. Environmental Responsibility

The supplier/contractor shall minimize environmental impact by:

- Proper disposal of construction waste

- Avoiding environmental contamination

- Maintaining cleanliness at the worksite

- Using environmentally responsible practices where feasible

9. Confidentiality

The supplier/contractor shall maintain confidentiality of all project-related documents, beneficiary information, and organizational data unless authorized in writing.

10. Conflict of Interest

The supplier/contractor shall disclose any actual or potential conflict of interest that may influence the tendering process or implementation of the contract.

11. Anti-Corruption and Anti-Bribery

The supplier/contractor shall not offer, give, request, or accept any gift, payment, commission, or benefit intended to influence procurement decisions or project activities.

12. Compliance Monitoring

CARE World reserves the right to monitor compliance with this Code of Conduct during the contract period. Non-compliance may result in:

- Disqualification from bidding

- Suspension or termination of contract

- Blacklisting from future procurement opportunities

- Legal action where applicable

Declaration by Supplier/Contractor

I/We hereby confirm that we have read, understood, and agree to comply with the above Supplier/Contractor Code of Conduct and all applicable laws and regulations.

| Name of Firm |  |
| --- | --- |
| Authorized Representative |  |
| Signature & Stamp |  |
| Date |  |